RISK ASSESSMENT & METHOD STATEMENT

DEMOLITION WORKS

Document Ref: RAMS-DM-001  |  Version: 1.0

| Company Name: | Project Name: | Site Address: | Client: |
| --- | --- | --- | --- |
| Date Prepared: | Review Date: | Prepared By: | Approved By: |
| ____/____/________ | ____/____/________ |  |  |

1. SCOPE OF WORKS

Description of Works:

Demolition of structures including soft strip, structural demolition, and site clearance. All works to comply with BS 6187:2011 Code of Practice for Full and Partial Demolition.

Activities Covered:

•  Soft strip and internal removal

•  Structural surveys and assessment

•  Disconnection of services

•  Structural demolition (mechanical)

•  Crushing and processing

•  Waste segregation and removal

•  Site remediation

2. RISK MATRIX

| L \ S | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 5 | 5 | 10 | 15 | 20 | 25 |
| 4 | 4 | 8 | 12 | 16 | 20 |
| 3 | 3 | 6 | 9 | 12 | 15 |
| 2 | 2 | 4 | 6 | 8 | 10 |
| 1 | 1 | 2 | 3 | 4 | 5 |

Severity (columns): 1=Negligible, 2=Minor, 3=Moderate, 4=Major, 5=Catastrophic

Likelihood (rows): 1=Rare, 2=Unlikely, 3=Possible, 4=Likely, 5=Almost Certain

3. HAZARD IDENTIFICATION & RISK ASSESSMENT

| Hazard | Who at Risk | Initial Risk (LxS) | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Premature/uncontrolled collapse | All persons in vicinity | 3x5=15 | Structural survey, demolition sequence planned by competent engineer, exclusion zones maintained | 1x5=5 |
| Falls from height | Operatives | 4x4=16 | Edge protection, harnesses where edge protection not possible, safe access routes | 2x4=8 |
| Asbestos exposure | All on site | 3x5=15 | R&D survey completed, licensed contractor for ACM removal, air monitoring | 1x5=5 |
| Dust generation | Operatives, neighbours | 4x3=12 | Water suppression, dust screens, RPE provided (FFP3), monitoring | 2x3=6 |
| Noise and vibration | Operatives, neighbours | 4x3=12 | Hearing protection mandatory, vibration monitoring, neighbourhood notification | 2x3=6 |
| Falling debris | All site personnel | 4x4=16 | Exclusion zones, debris netting, hard hats mandatory, controlled sequence | 2x4=8 |

4. METHOD STATEMENT

Sequence of Operations:

1.  Pre-demolition survey: structural assessment, asbestos survey, service disconnections confirmed

2.  Soft strip: remove all non-structural elements, fixtures, fittings, and services

3.  Establish exclusion zones and protect adjacent structures/services

4.  Demolition sequence: work from top down, maintain structural stability

5.  Mechanical demolition using long-reach excavator or similar plant

6.  Process and segregate materials on site where permitted

7.  Load and remove waste to licensed facilities

8.  Site clearance and making good

5. PPE REQUIREMENTS

•  Hard Hat (EN 397)

•  Hi-Vis Vest (EN 471 Class 2)

•  Safety Boots (EN 345 S3)

•  Gloves (EN 388)

•  Eye Protection (EN 166)

•  Ear Protection (mandatory)

•  Dust Mask (FFP3)

•  Full body harness (when at height)

6. EMERGENCY PROCEDURES

First Aider: ________________________   First Aid Kit Location: ________________________

Emergency Assembly Point: ________________________

Emergency Contacts:

•  Emergency Services: 999

•  Site Manager: ________________________

•  Company Office: ________________________

•  Nearest A&E: ________________________

In case of unexpected collapse: evacuate area immediately. Call 999. Account for all personnel. Do NOT re-enter until structural engineer confirms safe.

7. SIGN-OFF

I confirm that I have read, understood, and will comply with this RAMS:

| Name (Print) | Company | Signature | Date |
| --- | --- | --- | --- |

This template must be adapted to your specific site conditions and activities.

Template provided by Site Samurai — sitesamurai.co.uk