RISK ASSESSMENT & METHOD STATEMENT

EXCAVATION WORKS

Document Ref: RAMS-EX-001  |  Version: 1.0

| Company Name: | Project Name: | Site Address: | Client: |
| --- | --- | --- | --- |
| Date Prepared: | Review Date: | Prepared By: | Approved By: |
| ____/____/________ | ____/____/________ |  |  |

1. SCOPE OF WORKS

Description of Works:

Excavation operations including trenching, pits, shafts, and bulk earthworks. All excavations over 1.2m deep require formal support design. Compliant with CDM 2015 and BS 6031.

Activities Covered:

•  Trench excavation for services

•  Foundation excavation

•  Bulk earthworks and cut/fill

•  Trial pits and investigation holes

•  Installation of temporary works (shoring/trench sheets)

•  Backfilling, compaction, and reinstatement

2. RISK MATRIX

| L \ S | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 5 | 5 | 10 | 15 | 20 | 25 |
| 4 | 4 | 8 | 12 | 16 | 20 |
| 3 | 3 | 6 | 9 | 12 | 15 |
| 2 | 2 | 4 | 6 | 8 | 10 |
| 1 | 1 | 2 | 3 | 4 | 5 |

Severity (columns): 1=Negligible, 2=Minor, 3=Moderate, 4=Major, 5=Catastrophic

Likelihood (rows): 1=Rare, 2=Unlikely, 3=Possible, 4=Likely, 5=Almost Certain

3. HAZARD IDENTIFICATION & RISK ASSESSMENT

| Hazard | Who at Risk | Initial Risk (LxS) | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Collapse of excavation sides | Operatives in/near excavation | 4x5=20 | Shoring or battering to safe angle, competent person design, daily inspections, no materials stored at edge | 2x5=10 |
| Persons falling into excavation | All site personnel | 3x4=12 | Barriers, edge protection, adequate lighting, covered when unattended | 2x4=8 |
| Striking underground services | Operatives | 3x5=15 | Service plans obtained, CAT & Genny scans, hand dig near known services, safe dig practices | 1x5=5 |
| Water ingress/flooding | Operatives | 3x3=9 | Pump available, weather monitoring, dewatering plan, evacuate if rising water | 2x3=6 |
| Toxic/oxygen-deficient atmosphere | Operatives | 2x5=10 | Gas monitoring for deep excavations, forced ventilation if required, rescue plan | 1x5=5 |
| Plant operating near edge | Operatives in excavation | 3x5=15 | Stop blocks, minimum distance from edge, banksman in attendance, ground stability check | 1x5=5 |

4. METHOD STATEMENT

Sequence of Operations:

1.  Obtain service drawings and carry out CAT & Genny surveys before breaking ground

2.  Establish exclusion zones and barriers around the excavation area

3.  Excavate in stages — install temporary works as excavation deepens

4.  Hand dig within 500mm of any known services

5.  Competent person to inspect excavation daily and after heavy rain, frost, or vibration

6.  Ensure safe access and egress at all times (ladder every 25m)

7.  Backfill in layers, compact to specification, remove temporary works safely

8.  Reinstate surface and remove barriers

5. PPE REQUIREMENTS

•  Hard Hat (EN 397)

•  Hi-Vis Vest (EN 471 Class 2)

•  Safety Boots (EN 345 S3)

•  Gloves (EN 388)

•  Eye Protection (when breaking out)

•  Ear Protection (near plant)

•  Gas monitor (for deep excavations)

6. EMERGENCY PROCEDURES

First Aider: ________________________   First Aid Kit Location: ________________________

Emergency Assembly Point: ________________________

Emergency Contacts:

•  Emergency Services: 999

•  Site Manager: ________________________

•  Company Office: ________________________

•  Nearest A&E: ________________________

In case of excavation collapse with person buried: call 999 immediately. Do NOT enter collapsed excavation. Begin careful hand excavation from safe position if visible. Account for all personnel.

7. SIGN-OFF

I confirm that I have read, understood, and will comply with this RAMS:

| Name (Print) | Company | Signature | Date |
| --- | --- | --- | --- |

This template must be adapted to your specific site conditions and activities.

Template provided by Site Samurai — sitesamurai.co.uk