RISK ASSESSMENT & METHOD STATEMENT

HOT WORKS

Document Ref: RAMS-HW-001  |  Version: 1.0

| Company Name: | Project Name: | Site Address: | Client: |
| --- | --- | --- | --- |
| Date Prepared: | Review Date: | Prepared By: | Approved By: |
| ____/____/________ | ____/____/________ |  |  |

1. SCOPE OF WORKS

Description of Works:

Hot works including welding, cutting, grinding, soldering, and use of open flames or heat-producing equipment. All hot works require a valid Hot Works Permit before commencing.

Activities Covered:

•  MIG/TIG/arc welding

•  Oxy-acetylene cutting and brazing

•  Angle grinding and disc cutting

•  Soldering and brazing

•  Bitumen/asphalt heating

•  Use of heat guns and blow torches

2. RISK MATRIX

| L \ S | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 5 | 5 | 10 | 15 | 20 | 25 |
| 4 | 4 | 8 | 12 | 16 | 20 |
| 3 | 3 | 6 | 9 | 12 | 15 |
| 2 | 2 | 4 | 6 | 8 | 10 |
| 1 | 1 | 2 | 3 | 4 | 5 |

Severity (columns): 1=Negligible, 2=Minor, 3=Moderate, 4=Major, 5=Catastrophic

Likelihood (rows): 1=Rare, 2=Unlikely, 3=Possible, 4=Likely, 5=Almost Certain

3. HAZARD IDENTIFICATION & RISK ASSESSMENT

| Hazard | Who at Risk | Initial Risk (LxS) | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Fire and explosion | All in vicinity | 3x5=15 | Hot works permit, combustibles removed/protected, fire extinguisher on hand, 60-min fire watch after | 1x5=5 |
| Burns | Operatives | 4x3=12 | Flame-retardant clothing, welding gloves, face shield, screens for bystanders | 2x3=6 |
| Fume inhalation | Operatives | 4x3=12 | LEV where possible, RPE (FFP3 for welding fume), adequate ventilation | 2x3=6 |
| UV/arc eye exposure | Operatives, bystanders | 3x3=9 | Welding screens, correct shade welding lens, warning signs posted | 1x3=3 |
| Electric shock (welding) | Operatives | 3x4=12 | Equipment inspected, dry conditions, insulating mat, gloves | 1x4=4 |
| Gas cylinder hazards | All in vicinity | 2x5=10 | Cylinders secured upright, flashback arrestors fitted, regulators checked, storage away from heat | 1x5=5 |

4. METHOD STATEMENT

Sequence of Operations:

1.  Apply for and obtain Hot Works Permit from site manager before starting

2.  Clear area of combustible materials — minimum 10m radius

3.  Protect surfaces that cannot be removed with fire-retardant blankets/sheets

4.  Position fire extinguisher within arm reach of hot works area

5.  Erect welding screens to protect others from sparks and UV

6.  Check ventilation is adequate — use LEV if enclosed space

7.  Complete hot works, extinguish all ignition sources

8.  Maintain fire watch for minimum 60 minutes after completion

5. PPE REQUIREMENTS

•  Welding helmet (correct shade lens)

•  Flame-retardant overalls/jacket

•  Welding gauntlets (EN 12477)

•  Safety Boots (EN 345 S3)

•  Leather apron (for extended welding)

•  Respiratory protection (FFP3 for welding fume)

•  Hi-Vis Vest (when not welding)

6. EMERGENCY PROCEDURES

First Aider: ________________________   First Aid Kit Location: ________________________

Emergency Assembly Point: ________________________

Emergency Contacts:

•  Emergency Services: 999

•  Site Manager: ________________________

•  Company Office: ________________________

•  Nearest A&E: ________________________

In case of fire: raise alarm, evacuate area, call 999. Use fire extinguisher ONLY if safe and trained to do so. If gas leak suspected: eliminate ignition sources, ventilate area, evacuate.

7. SIGN-OFF

I confirm that I have read, understood, and will comply with this RAMS:

| Name (Print) | Company | Signature | Date |
| --- | --- | --- | --- |

This template must be adapted to your specific site conditions and activities.

Template provided by Site Samurai — sitesamurai.co.uk