RISK ASSESSMENT & METHOD STATEMENT

LIFTING OPERATIONS

Document Ref: RAMS-LO-001  |  Version: 1.0

| Company Name: | Project Name: | Site Address: | Client: |
| --- | --- | --- | --- |
| Date Prepared: | Review Date: | Prepared By: | Approved By: |
| ____/____/________ | ____/____/________ |  |  |

1. SCOPE OF WORKS

Description of Works:

Lifting operations using cranes, telehandlers, hoists, and other lifting equipment. All operations to comply with LOLER 1998 and BS 7121. A Lift Plan is required for all lifting operations.

Activities Covered:

•  Mobile crane lifts

•  Telehandler/forklift operations

•  Hoist operations

•  Use of lifting accessories (slings, shackles, chains)

•  Tandem lifts (if applicable)

•  Personnel lifting (MEWP)

2. RISK MATRIX

| L \ S | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 5 | 5 | 10 | 15 | 20 | 25 |
| 4 | 4 | 8 | 12 | 16 | 20 |
| 3 | 3 | 6 | 9 | 12 | 15 |
| 2 | 2 | 4 | 6 | 8 | 10 |
| 1 | 1 | 2 | 3 | 4 | 5 |

Severity (columns): 1=Negligible, 2=Minor, 3=Moderate, 4=Major, 5=Catastrophic

Likelihood (rows): 1=Rare, 2=Unlikely, 3=Possible, 4=Likely, 5=Almost Certain

3. HAZARD IDENTIFICATION & RISK ASSESSMENT

| Hazard | Who at Risk | Initial Risk (LxS) | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Crane/plant overturn | Operator, persons nearby | 2x5=10 | Lift plan prepared, ground conditions assessed, outriggers on spreader plates, load within SWL | 1x5=5 |
| Dropped load | All in lift zone | 3x5=15 | Exclusion zone, banksman/slinger, SWL checks, equipment inspected (LOLER 6-monthly) | 1x5=5 |
| Contact with overhead power lines | Operator, banksman | 2x5=10 | Minimum clearance 15m, goal posts if needed, electricity company notified | 1x5=5 |
| Crushing/trapping | Slinger/banksmen | 3x4=12 | Tag lines used, no hands on load during slewing, safe positioning during landing | 2x4=8 |
| Failure of lifting accessories | All in lift zone | 2x5=10 | Accessories examined (LOLER), SWL marked, correct selection for load | 1x5=5 |
| Wind loading | All in lift zone | 3x4=12 | Wind speed monitoring, abort criteria defined in lift plan, anemometer on crane | 2x4=8 |

4. METHOD STATEMENT

Sequence of Operations:

1.  Prepare lift plan including load weight, radius, SWL, ground conditions, exclusion zones

2.  Brief all personnel involved — operator, slinger/banksmen, site team

3.  Inspect all lifting equipment and accessories — confirm current examination certificates

4.  Set up crane/plant on firm level ground, deploy outriggers with spreader plates

5.  Establish exclusion zone and position banksman with clear line of sight

6.  Trial lift if required — check stability and sling configuration

7.  Carry out lift — smooth movements, no sudden stops, follow banksman signals

8.  Land load safely, disconnect when fully supported, record completed lift

5. PPE REQUIREMENTS

•  Hard Hat (EN 397) with chin strap

•  Hi-Vis Vest (EN 471 Class 3)

•  Safety Boots (EN 345 S3)

•  Gloves (EN 388)

•  Two-way radio (for communication during lift)

6. EMERGENCY PROCEDURES

First Aider: ________________________   First Aid Kit Location: ________________________

Emergency Assembly Point: ________________________

Emergency Contacts:

•  Emergency Services: 999

•  Site Manager: ________________________

•  Company Office: ________________________

•  Nearest A&E: ________________________

In case of crane failure or load drop: evacuate exclusion zone. Call 999 if persons injured. Isolate crane — do NOT attempt to re-rig or recover without competent person assessment.

7. SIGN-OFF

I confirm that I have read, understood, and will comply with this RAMS:

| Name (Print) | Company | Signature | Date |
| --- | --- | --- | --- |

This template must be adapted to your specific site conditions and activities.

Template provided by Site Samurai — sitesamurai.co.uk