RISK ASSESSMENT & METHOD STATEMENT

WORKING AT HEIGHT

Document Ref: RAMS-WAH-001  |  Version: 1.0

| Company Name: | Project Name: | Site Address: | Client: |
| --- | --- | --- | --- |
| Date Prepared: | Review Date: | Prepared By: | Approved By: |
| ____/____/________ | ____/____/________ |  |  |

1. SCOPE OF WORKS

Description of Works:

All work activities requiring personnel to work at height, including use of scaffolding, mobile elevated work platforms (MEWPs), ladders, and roof work. Compliant with Work at Height Regulations 2005.

Activities Covered:

•  Scaffold-based work activities

•  MEWP operations

•  Ladder access (short duration tasks)

•  Roof work and edge protection

•  Installation of fall protection systems

•  Access tower assembly and use

2. RISK MATRIX

| L \ S | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 5 | 5 | 10 | 15 | 20 | 25 |
| 4 | 4 | 8 | 12 | 16 | 20 |
| 3 | 3 | 6 | 9 | 12 | 15 |
| 2 | 2 | 4 | 6 | 8 | 10 |
| 1 | 1 | 2 | 3 | 4 | 5 |

Severity (columns): 1=Negligible, 2=Minor, 3=Moderate, 4=Major, 5=Catastrophic

Likelihood (rows): 1=Rare, 2=Unlikely, 3=Possible, 4=Likely, 5=Almost Certain

3. HAZARD IDENTIFICATION & RISK ASSESSMENT

| Hazard | Who at Risk | Initial Risk (LxS) | Control Measures | Residual Risk |
| --- | --- | --- | --- | --- |
| Falls from height | Operatives | 4x5=20 | Guard rails, toe boards, harness and lanyard where edge protection not feasible, rescue plan in place | 2x5=10 |
| Falling objects/tools | All below | 3x4=12 | Toe boards, tool lanyards, exclusion zone below, debris netting | 2x4=8 |
| Scaffold collapse | Operatives, public | 2x5=10 | Designed by competent person, weekly inspections, load limits observed | 1x5=5 |
| MEWP overturn | Operator | 2x5=10 | Level ground, outriggers deployed, wind speed monitoring, trained operator | 1x5=5 |
| Ladder slip/fall | Operatives | 3x4=12 | Ladder secured at top and bottom, 1:4 ratio, 3-point contact, short duration only | 2x4=8 |
| Fragile surfaces | Operatives | 3x5=15 | Fragile roof covers/platforms, warning signs, crawling boards, harness | 2x5=10 |

4. METHOD STATEMENT

Sequence of Operations:

1.  Assess whether work at height can be avoided or carried out from ground level

2.  Select appropriate access equipment for the task and duration

3.  Ensure all operatives are trained and competent for the access equipment to be used

4.  Erect scaffolding/platforms — inspect and tag before use

5.  Establish edge protection and exclusion zones below

6.  Brief all operatives on emergency rescue procedures

7.  Carry out work maintaining three points of contact where applicable

8.  Dismantle access equipment safely upon completion

5. PPE REQUIREMENTS

•  Hard Hat (EN 397) with chin strap

•  Hi-Vis Vest (EN 471 Class 2)

•  Safety Boots (EN 345 S3 — non-slip sole)

•  Gloves (EN 388)

•  Full Body Harness (EN 361) with lanyard

•  Tool lanyards for hand tools

•  Eye Protection (when cutting/drilling)

6. EMERGENCY PROCEDURES

First Aider: ________________________   First Aid Kit Location: ________________________

Emergency Assembly Point: ________________________

Emergency Contacts:

•  Emergency Services: 999

•  Site Manager: ________________________

•  Company Office: ________________________

•  Nearest A&E: ________________________

Rescue plan must be in place before work begins. If a person is suspended in a harness, rescue within 15 minutes to prevent suspension trauma. Ensure trained rescue personnel and equipment are available.

7. SIGN-OFF

I confirm that I have read, understood, and will comply with this RAMS:

| Name (Print) | Company | Signature | Date |
| --- | --- | --- | --- |

This template must be adapted to your specific site conditions and activities.

Template provided by Site Samurai — sitesamurai.co.uk